SAFETY DATA SHEET

ARVAK

SECTION 1: IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

| 1.1. | Product identifier | ARVAK Contains iprodione |
| --- | --- | --- |
| 1.2. | Relevant identified uses of the substance or mixture and uses advised against | Can be used as fungicide only. |
| 1.3. | Details of the supplier of the safety data sheet | Cheminova Agro, S.A. Paseo de la Castellana, 257 Madrid, Spain buzon@cheminova.com |
| 1.4. | Emergency telephone number | +34 915620420 (24 h; for emergencies only) |

SECTION 2: HAZARDS IDENTIFICATION

| 2.1. | Classification of the substance or mixture | See section 16 for full text of R-phrases and hazard statements. |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DPD classification of the product according to Dir. 1999/45/EC as amended | R40; R51/53 |  |  |  |
|  | CLP classification of the product according to Reg. 1272/2008 as amended | Hazards to the aquatic environment: Chronic Category 2 (H411) |  |  |  |
|  | WHO classification Guidelines to Classification 2009 | Class U (Unlikely to present acute hazard in normal use) |  |  |  |
|  | Health hazards | Limited evicence of carcinogenic efffects |  |  |  |
|  | Environmental hazards | The product is harmful to aquatic organisms. |  |  |  |
| 2.2. | Label elements |  |  |  |  |
|  | Labelling in Greece |  |  |  |  |
|  | Hazard symbols | Xn | N |  |  |
|  |  | Harmful Dangerous for the environment |  |  |  |
|  | R-phrase R40 R51/53 | Limited evidence of a carcinogenic effect Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |  |  |  |
|  | S-phrases S1/2 S13 S20/21 S22 S24/25 S36/37 S60/61 SP1 SPe3 | Keep locked up and out of the reach of children. Keep away from food, drink and animal feedingstuffs. When using do not eat, drink or smoke. Do not breathe dusts. Avoid contact with skin and eyes Wear suitable protective clothing and gloves. This material and its container must be disposed of as harzadous waste. Avoid release to the environment. Refer to special instructions/safety data sheets. Do not contaminate water with the product or its container (Do not clean application equipment near surface water/Avoid contamination via drains from farmyards and roads). To protect aquatic organisms/non-target plants/non-target arthropods/insects respect an unsprayed buffer zone of 10 m to non-agricultural land/surfact water bodies. |  |  |  |

|  | According to EU Reg. 1272/2008 as amended |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Product identifier | ARVAK 50 WG |  |  |  |
|  |  | Contains iprodione |  |  |  |
|  | Hazard pictograms |  |  |  |  |
|  | Signal word | Danger |  |  |  |
|  | Hazard statement |  |  |  |  |
|  | H351 | Suspected of causing cancer. |  |  |  |
|  | Supplementary hazard statement EUH401 Supplementary phrase for final use of the product for plant protection: SP1 SPe3 | To avoid risks to human health and the environment, comply with the instructions of use. Do not contaminate water with the product or its container (Do not clean application equipment near surface water/Avoid contamination via drains from farmyards and roads). To protect aquatic organisms/non-target plants/non-target arthropods/insects respect an unsprayed buffer zone of 10 m to non-agricultural land/surfact water bodies. |  |  |  |
|  | Precautionary statements |  |  |  |  |
|  | P201 P202 P281 P308 + P 313 P405 P501 | Obtain special instructions before use Do not handle until all safety precautions have been read and understood. Use personal protective equipment as required IF exposed or concerned: Get medical attention/advice Store locked up Dispose of contents/container as hazardous waste. |  |  |  |
| 2.3. | Other hazards | None of the ingredients meets the criteria for being PBT or vPvB. |  |  |  |

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

| 3.1. | Substances | The product is a mixture, not a substance. |
| --- | --- | --- |
| 3.2. | Mixtures | See section 16 for full text of R-phrases and hazard statements. |
|  | Active ingredient |  |
|  | Iprodione | Content: 50% by weight |
|  | CAS name/EU name | 3-(3,5-Dichlorophenyl)-N-(1-methylethyl)2,4-dioxo-1-imida zolidinecarboxamide; Rovral; 3-(3,5-dichlorophenyl)-2,4-dioxo-N-(propan-2-yl)imidazolidine-1-carboxamide; 3-(3,5-dimethylphenyl)-N-isopropyl-2,4-dioxo-imidazolidine-1-carboxamide |
|  | CAS no. | 36734-19-7 |
|  | IUPAC name(s) | 3-(3,5-dichlorophenyl)-N-isopropyl-2,4-dioxoimidazolidine-1-carboxamide |
|  | ISO name | Iprodione |
|  | EU name | Glycophene |
|  | EC no. (EINECS no.) | 253-178-9 |
|  | EU index no. | 616-054-00 -9 |
|  | DSD classification of the ingredient | Xn, N; R40, R50/53 |
|  | CLP classification of the ingredient | Carc. 2 (H351) Hazards to the aquatic environment: Aquatic Acute 1 (H400) Aquatic Chronic 1 (H410) |
|  | Structural formula |  |
|  | Reportable ingredient |  |

|  |  | Content (% w/w) | CAS no. | EC no. (EINECS no.) | DSD classification | CLP classification |
| --- | --- | --- | --- | --- | --- | --- |
|  | Diisopropyl naphthalen sodium Sulfonate sodium methylnaphthalen + formaldehyde condensate | 2% 8% | 1322-93-6 81065-51-2 |  | Xn;R20/22 R36/37 Harmful Xi; R41 | Not yet available Not yet available |

SECTION 4: FIRST AID MEASURES

| 4.1. | Description of first aid measures |  |
| --- | --- | --- |
|  | Inhalation | Remove victim from exposure area or contaminated zone. Maintain airway, if vomiting occurs. If necessary, apply artificial respiration. If victim is in a state of shock, keep at rest and cover with a blanket. Seek medical attention immediately. Symptomatic and supportive treatment |
|  | Skin contact | Remove victim clothing contaminated with the product immediately. Wash the affected body zones with abundant water and soap (if there are no burns), avoiding rubbing these zones. Seek medical attention immediately. |
|  | Eye contact | Wash the eyes with abundant water or saline solution at least during 15 minutes with the eyelids open. If poisoning symptoms ocurrs, seek immediate oftalmological medical attention . |
|  | Ingestion | Do not provoke vomit. Chemical pneumonia caused by aspiration. Do not administer anything by mouth. Seek medical attention immediately. Show label, container or this SDS whenever possible. |
| 4.2. | Most important symptoms and effects, both acute and delayed | Low acute oral and dermal toxicity |
| 4.3. | Indication of any immediate medical attention and special treatment needed | Immediate medical attention is required in case of ingestion. It may be helpful to show this safety data sheet to physician. |
|  | Note to physician | No first aid available for this particular chemical. Get medical aid. |

SECTION 5: FIRE-FIGHTING MEASURES

| 5.1. | Extinguishing media | Carbon Dioxide, dry chemical powder, foam and water spray. Do not use water pressure jet. |
| --- | --- | --- |
| 5.2. | Special hazards arising from the substance or mixture | Move containers away from fire area if possible. Cool the drums/containers by water spraying and in case there is an explosion keep a security distance. Do not scatter spilled material with high pressure water streams. Dike and collect water and other materials used to fight fire for later disposal. Avoid breathing harmful fumes or vapours in case of fire. Keep upwind and fight the fire from protected places. Wear full protective equipment for fire extinction: Suitable breathing apparatus and protective clothing. Combustion products: Thermal decomposition may include toxic and hazardous vapours of Carbon and Nitrogen oxides, Hydrogen chloride. |
| 5.3. | Advice for firefighters | Use water spray to keep fire-exposed containers cool. Approach fire from upwind to avoid hazardous vapours and toxic decomposition products. Fight fire from protected location or maximum possible distance. Dike area to prevent water runoff. Firemen should wear self-contained breathing apparatus and protective clothing. |

SECTION 6: ACCIDENTAL RELEASE MEASURES

| 6.1. | Personal precautions, protective equipment and emergency procedures | Keep upwind. Avoid breathing sprays, vapours and dust. Do not use contact lenses. Avoid contact with skin and eyes. Keep unnecessary people away. Adequate ventilation/breathing device. Wear suitable protective clothing. Decontaminate emergency personnel with soap and water before leaving the release area. |
| --- | --- | --- |
| 6.2. | Environmental precautions | Isolate and dyke release or fire control water area to prevent material run-off and to avoid contamination of superficial waters and the entry of the product into the water course or the sewers network as well as in zones with vegetation, since the product is toxic to aquatic organisms. Wash contaminated area with water and soap and collect washing waters in dry, clean suitable containers for later disposal. In case of spillage arrival into the sewage network system or the water courses, warn and notify immediately to the competent authorities. |
| 6.3. | Methods and materials for containment and cleaning up | Collect spilled material using an absorbing material or sand. Do not touch spilled material. Stop spillage if this can be done without risk. Collect spilled material in dry, clean suitable containers, hermetically closed and take it away from spilled area for disposal according with the local provisions for industrial residue management. Wash contaminated area with water and soap and collect washing waters in dry, clean suitable containers for later disposal. Decontaminate all used tools, equipment and cleaning material. |
| 6.4. | Reference to other sections | See subsection 8.2. for personal protection. See section 13 for disposal. |

SECTION 7: HANDLING AND STORAGE

| 7.1. | Precautions for safe handling | Avoid contact with eyes, skin or clothes. Use protective clothes and observe normal precautions for the handling of chemical products. Precautions in case of fire and explosion hazard: Keep and handle the product away from sparks, open flame and excessive heat. Take measures to prevent the build up of electrostatic charge. Avoid dust formation. Dusts and air mixtures can be dangerous. Get chemical fire extinguishers at hand. Do not contaminate waters with the product or its container nor cleaning spraying devices near superficial or ground waters. Avoid contamination or spills from entering into road farm pipes, sewage system or droves. Avoid contamination by material run-off, since the product is toxic to aquatic organisms. Do not breathe dusts. Use appropriate natural or forced ventilation system in handling areas. |
| --- | --- | --- |
|  |  | Do not discharge to the environment. Collect all waste material and remains from cleaning equipment, etc., and dispose of as hazardous waste. See section 13 for disposal. |
| 7.2. | Conditions for safe storage, including any incompatibilities | Store in tightly closed containers in a cool, dry, well-ventilated area out of the reach of the children. Preserve of humidity. Do not store together with incompatible materials, strong oxidisers or in conditions of excessive heat. Do not store into the houses. Store it in a restricted and locked area. Keep out of food, feed and drinks. Stable under normal conditions storage at least two years if stores in its original container unopened and at room temperature. Protect from extreme temperatures. Do not eat, drink nor smoke in places where pollution with this product is possible. Keep out from contact with water, fertilizers, food, feed or forage. |
| 7.3. | Specific end use(s) | The product is a registered pesticide which may only be used for the applications it is registered for, in accordance with a label approved by the regulatory authorities. |

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

| 8.1. | Control parameters |  |  |
| --- | --- | --- | --- |
|  | Personal exposure limits |  | No Occupational Exposure limit values established by the OSHA, NIOSH or ACGIH for Iprodione (Std OE~2 mg/m3). |
| 8.2. | Exposure controls |  | Risk evaluation General Occupational Health protection measures: Keep clean the working place. Avoid skin, eyes and clothes contact with the product. Keep and wash working clothes and PPE separately from other laundry. Change clothes if these are soiled with the product. Wash after handling, especially hands and parts of the body which may have been exposed. Launder clothes separately before reuse. Install safety showers and eye-washing facilities. Do not smoke, eat or drink during handling. |
|  |  | Respiratory protection | Respiratory mask with filter for solid and liquid particles of harmful substances: EN 143 P2 medium, when loading/unloading, mixing spraying and disposal of pesticide mix. Wear it when air purifiers does not fit adequate protection. In case of fire, wear a full-face piece self-contained breathing apparatus. Ensure good general ventilation system to reduce air concentration level. |
|  |  | Protective gloves | Wear homologated waterproof gloves, resistant to chemical products: Neoprene, nitrile- Rubber, butyl- Rubber, barrier laminate. |
|  |  | Eye protection | To protect against accidental eye contact, safety glasses or goggles with side-shields, face-shield should be worn. Eye protection should conform to EN166 or local equivalent. Avoid using contact lenses. |
|  |  | Other skin protection | Appropriate waterproof protective suit, clothing and apron which do not accumulate dust: Long-sleeved overalls with chemical-resistant rubber boots and socks(with the trousers on the boots). |
|  |  |  | Appropriate waterproof protective suit, clothing and apron which do not accumulate dust: Long-sleeved overalls with chemical-resistant rubber boots and socks(with the trousers on the boots). |

Environmental exposure controls: 	Do not apply immediately after rain or irrigation, when plants are covered with dew or in very dry conditions or high temperatures. Very toxic to aquatic organisms. May cause long-term effects in the aquatic environment. Avoid the contamination of waters. Avoid spillage arrival into the sewage network system or the  water courses. In case of water contamination, warn and notify immediately to the competent authorities.

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

| 9.1. | Information on physical and chemical properties |  |
| --- | --- | --- |
|  | Appearance | White-beig solid dispersible granules |
|  | Odour | Characteristic |
|  | Odour threshold | Not determined |
|  | pH | 5-7 (CIPAC MT-75 at 1% acqueous solution in CIPAC water D). |
|  | Initial boiling point and boiling range | No data available |
|  | Flash point | Not flammable |
|  | Evaporation rate | Not applicable |
|  | Flammability (solid/gas) | Not flammable |
|  | Vapour pressure | No data available |
|  | Vapour density | Not applicable |
|  | Relative density | No data available |
|  | Solubility(ies) | Hydrosolubility: Insoluble in water. Dispersible. Liposolubility (solvent oil to be specified): Iprodione Tech. at 20 ºC Hexane 0.59g/L, Toluene 147 Dichloromethane 450 Acetonitrile 168 Acetone 342 Ethyl-acetate 225. |
|  | Partition coefficient n-octanol/water | K ow log P =3 (Iprodione tech.) |
|  | Viscosity | Not applicable |
|  | Explosive properties | Not considered to be explosive |
|  | Oxidising properties | Not oxidising (EEC A17) 128-134 oC (Iprodione tech.) |
| 9.2. | Other information |  |
|  | Melting point: | 128-134 oC (Iprodione tech.) |

SECTION 10: STABILITY AND REACTIVITY

| 10.1. | Reactivity | No hazardous reactions nor polymerization occurs. |
| --- | --- | --- |
| 10.2. | Chemical stability | Stable at normal pressure and temperature conditions. Strong oxidizers and high temperatures decomposes the product with emission of toxic fumes. Photo stability in water depends on pH: DT50 ~ 67 d at pH 5 and 25 °C; DT50: < 1 h at pH 9 simulated solar light conditions. It degrades by UV light, specially in aqueous media: DT50 range between 5-10 minutes. Store in dry and cool conditions. Stable in acidic conditions. Decomposes in alkaline conditions. |
| 10.3. | Possibility of hazardous reactions | No hazardous reactions nor polymerization occurs. Photo stability in water depends on pH: DT50 ~ 67 d at pH 5 and 25 °C; DT50: < 1 h at pH 9 simulated solar light conditions. It degrades by UV light, specially in aqueous media: DT50 range between 5-10 minutes. Store in dry and cool conditions. Stable in acidic conditions. Decomposes in alkaline conditions. |
| 10.4. | Conditions to avoid | Avoid high temperatures and contact with heat sources, open flame or other sources of ignition. Avoid dust inhalation and contact with eyes and skin since active substance is harmful. Avoid water contamination since active substance is very toxic for aquatic organisms. |
| 10.5. | Incompatible materials | Incompatibility with strong acid, bases and oxidizing agents. |
| 10.6. | Hazardous decomposition products | High temperatures can produce toxic and corrosive smokes of Carbon and Nitrogen oxides(COx, NOx), Hydrogen chloride, chlorine. |

SECTION 11: TOXICOLOGICAL INFORMATION

| 11.1. | Information on toxicological effects |  |
| --- | --- | --- |

| Acute toxicity | ###### Iprodione Tech. | Iprodione 50 % WDG |
| --- | --- | --- |
| Acute toxicity – ORAL LD 50 mg/kg b.w. (rats) | 2000 mg/kg | >2000 mg/kg |
| Acute toxicity – DERMAL LD 50 mg/kg b.w. (rats) | 2500 mg/kg | > 2 000 mg/kg |
| Acute toxicity – INHALATION LC 50 4h. (rats) | >5.16 mg/L air | > 5.16 mg/L air |
| EYE IRRITATION | Non irritant | Non irritant |
| SKIN IRRITATION | Non Irritant | Non irritant |
| Skin sensitisation (M&K test) Guinea-pig | Non Sensitising (Buehler) | Non sensitising |

Delayed effects after exposure: [sensitisation, narcosis, carcinogenicity, mutagenicity, reproductive toxicity (developmental toxicity and fertility) ]

Immediate: Short term toxicity in mammals: NOEL (90d - rats): 300 ppm

Chronic:

| Chronic toxicity (Iprodione) |  |
| --- | --- |
| NOEL (rat) 90 d. oral | 31 mg/kg p.c./d |
| NOEL (rabbit) 21 d. dermal | 100 mg/kg p.c./d |
| NOAEL (rat) 2 years | 6.1 mg/kg p.c./d. |
| Carcinogenicity | Liver/ testicular/ovary benign Tumors in mice/rats |
| Mutagenicity | No mutagenic nor genotoxics effects known |
| Effects on reproduction | Delays in development an body weight |
| Sensitization | No |

Effects of exposure: Limited evidence of a carcinogenic effect. Iprodione is not mutagenic, not genotoxic nor differed neurotoxicant. Lower relevant reproductive NOAEL: 17 mg/kg b.w.  Lower relevant development NOAEL: 20 mg/kg b.w. Dermic absorptin (rat): 1% ADI: 0.06 mg/kg b.w. No poisoning nor adverse effects  in workers have been reported.

SECTION 12: ECOLOGICAL INFORMATION

| 12.1. | Ecotoxicity | Aquatic Toxicity: Very toxic to aquatic organisms. Acute: Toxicity to Fish: Rainbow trout: LC50 (96h.)= 4.1 mg a.s./L (Iprodione tech.) Bluegill sunfish: LC50 (96h.)= 3.7 mg a.s./L (Iprodione tech.) LC50 (other species)= 3.1-7.7 mg a.s./L (Iprodione tech.) LC50 (bluegill)= 0.55 mg /L (Metabolite RP30228) Toxicity Daphnia: Daphnia magna (48h.)EC50= 0.66 mg s.a./L. (Iprodione tech.) Toxicity Algae and other Aquatic Plants: Algae EC50: 1.8 mg a.s./l; EC50: (S. subspicatus) 0.5 mg /l (Metabolite RP 30228) Chronic: Not available Toxicity to Fish: Long term toxicity fish: NOEC (34 d.): 0.26 mg s.a./l Fish Bioaccumulation: BCF: 34.8 (edible); 46.8 (non edible); 70 (whole fish) Toxicity Daphnia: NOEC(21 days): 0.17 mg a.s./L. Toxicity Algae and other Aquatic Plants: Not relevant for Iprodione tech . NOEC 28 d (Chironomus riparius 100μg/l) organisms which live in sediments (Metabolite RP 30228) Toxicity to soil micro and macroorganisms: Microorganisms: Do not eliminate microflora nor its methabolic processes, except thoseat very high rates. It does not affect nitrogen fixation nor redox processes of nitrogen cycle. Nitrogen mineralization: No effect at 2 and 10 mg/kg soil. Carbon mineralization: No significative effect at 2 and 10 mg/kg soil. Macroorganisms: Acute toxicity to earthworms: CL50>1000 mg/kg soil. Iprodione tech. Reproductive Toxicity: NOEC= 1000 mg/kg soil (Iprodione tech. and RP30228). RP 32596 : NOEC = 100 mg/kg soil Toxicity to other important organisms for the environment: (peer review) Toxicity birds: DL50oral (mallard duck/bobwhite quail)dietary: >5620mg s.a./kg bw. Reproductive toxicity birds: NOEC 300 ppm (duck/quail) Iprodione. Toxicity bees: DL50 (48h) contact: >200 g a.s./bee. Iprodione// DL50 (24h) oral: 25 ga.s./bee // Field study in cereals: NOAEC= 20 g/ha Toxicity plants: Not available. |
| --- | --- | --- |
| 12.2. | Persistence and degradability | Degradation half life in Lab tests soils of DT50: 26-160 days(25-15ºC pH 6.8); in fields tests: 42-126 days ( pH 4.-8.58). Iprodione degradation half life in waters is dependent of pH DT50: 146 days(pH 5); DT50 3days (pH7) and DT50 0.2 days (pH8). No accumulation of Iprodione in waters. |
| 12.3. | Bioaccumulative potential | Iprodione does not provoke bioaccumulation in mammals. Rapid excretion by urine and faeces over 96 h. Not readily biodegradable log P ow = 2.99-3 |
| 12.4. | Mobility in soil | Low lixiviation in soils of Iprodione and its metabolites. Not significant. It is strongly adsorbed to soil and organic matter sediments. KOC: 202-543 (4 soils) |
| 12.5. | Results of PBT and vPvB assessment | None of the ingredients meets the criteria for being PBT or vPvB. |
| 12.6. | Other adverse effects | Other relevant hazardous effects in the environment are not known. |

SECTION 13: DISPOSAL CONSIDERATIONS

| 13.1. | Waste treatment methods Disposal of product | Remaining quantities of the material and empty but unclean packaging should be regarded as hazardous waste. According to the Waste Framework Directive (2008/98/EC), possibilities for reuse or reprocessing should first be considered. If this is not feasible, the material can be disposed of by removal to a |
| --- | --- | --- |
|  |  | licensed chemical destruction plant or by controlled incineration with flue gas scrubbing. |
|  |  | Do not contaminate water, foodstuffs, feed or seed by storage or disposal. Do not discharge to sewer systems. |
|  | Disposal of packaging | Containers can be triply rinsed (or equivalent) and offered for recycling or reconditioning. Alternatively, the packaging can be punctured to make it unusable for other purposes and then be disposed of in a sanitary landfill. Controlled incineration with flue gas scrubbing is possible for combustible packaging materials. Disposal of waste and packagings must always be in accordance with all applicable local regulations. |

SECTION 14: TRANSPORT INFORMATION

|  | ADR/RID/IMDG/IATA/ICAO classification |
| --- | --- |

| 14.1. | UN number | 3077 |
| --- | --- | --- |
| 14.2. | UN proper shipping name | Environmentally hazardous substance, solid, n.o.s. (Iprodione), 9, III |
| 14.3. | Transport hazard class(es) | 9 |
| 14.4. | Packing group | III |
| 14.5. | Environmental hazards | Marine pollutant |
| 14.6. | Special precautions for user | Do not discharge to the environment. |
| 14.7. | Transport in bulk according to Annex II of MARPOL 73/78 and the IBC code | The product is not transported in bulk tankers. |

SECTION 15: REGULATORY INFORMATION

| 15.1. | Safety, health and environmental regulations/legislation specific for the substance or mixture | Seveso category in Annex I, part 2, to Dir. 96/82/EC: dangerous for the environment. |
| --- | --- | --- |
| 15.2. | Chemical safety assessment | A chemical safety assessment has not been performed. |

SECTION 16: OTHER INFORMATION

|  | List of abbreviations | B.o.a.d.t.c.c.a.n.m.: Based on available data the classification criteria are not met. CAS Chemical Abstracts Service CLP Classification, Labelling and Packaging; refers to EU regulation 1272/2008 as amended Dir. Directive DNEL Derived No Effect Level DPD Dangerous Preparation Directive; refers to EU directive 1999/45/EC DSD Dangerous Substance Directive; refers to EU directive 67/548/EEC EC50 50% Effect Concentration EINECS European Inventory of Existing Commercial Chemical Substances GHS Globally Harmonized classification and labelling System of chemicals, Fourth revised edition 2011 IBC International Bulk Chemical code ISO International Organisation for Standardization IUPAC International Union of Pure and Applied Chemistry LC50 50% Lethal Concentration LD50 50% Lethal Dose MARPOL Set of rules from the International Maritime Organisation (IMO) for prevention of sea pollution NOAEL No Observed Adverse Effect Level NOEC No Observed Effect Concentration N.o.s. Not otherwise specified OECD Organisation for Economic Cooperation and Development PBT Persistent, Bioaccumulative, Toxic PE Polyethylene PNEC Predicted No Effect Concentration PROC Process Category QSAR Quantitative Structure-Activity Relationship Reg. Regulation SP Safety Precaution STOT Specific Target Organ Toxicity US-EPA Environmental Protection Agency USA vPvB very Persistent, very Bioaccumulative WDG Water Dispersible Granules |
| --- | --- | --- |
|  | References | Data measured on the product are unpublished company data. Data on ingredients are available from published literature and can be found several places. |
|  | Method for classification | Calculation rules |
|  | Used R-phrases | R20/22: Harmful by inhalation and if swallowed R36/37: Irritating to eyes and respiratory system R40: Limited evicence of carcinogenic efffects R41: Risk of serious damage to eyes R50/53: Very toxic to aquatic organisms. May cause long-term adverse effects in the aquatic environment. |
|  | Used CLP statements H351 P201 P202 P281 P308 + P 313 P405 P501 | Suspected of causing cancer. Obtain special instructions before use Do not handle until all safety precautions have been read and understood. Use personal protective equipment as required IF exposed or concerned: Get medical attention/advice Store locked up Dispose of contents/container as hazardous waste. |
|  | Advice on training | This material should only be used by persons who are made aware of its hazardous properties and have been instructed in the required safety precautions. |

|  | The information provided in this safety data sheet is believed to be accurate and reliable, but uses of the product vary and situations unforeseen by Cheminova Agro, S.A. may exist. The user has to check the validity of the information under local circumstances. |
| --- | --- |